N64 Repair Docs — Source Cheat Sheet

Nintendo of Latin America service training package, c. 1997  ·  Hard4Games leak, 2020

Three documents  ·  Oscilloscope captures dated 8 April 1997  ·  NUS-CPU-03 board (NTSC)

Document Overview

| Document | What it is |
| --- | --- |
| Communication & Signal (5 pages) | Oscilloscope captures from a live NUS-CPU-03 board, taken 8 April 1997. Six pages of scope shots with handwritten annotations in Spanish. Covers: RCP/cartridge bus, PIF/RCP communication, video signal chain from VDC bus to composite output, audio DAC chain, and all system clocks. |
| N64 Repair Manual (9 pages) | Nintendo of Latin America service manual — D.C.N. NUS-06-0014-001A, Rev. 02/21/97. Includes: NUS block diagram, start-up sequence, Case 1 / Case 2 / Case 3 diagnostic flow charts, and a TDS 380 oscilloscope setup guide with reference waveforms. |
| Procedure for Testing N64 (2 pages) | Handwritten technician training notes, likely from a NoL service course. Left column: Case 1 and Case 2 diagnostic procedures in shorthand. Right column: common fault patterns (13 entries) with root cause. Includes a fully annotated system schematic with technician markup in multiple colours. |

ℹ Language note The handwritten annotations are in Spanish. Key terms used throughout: Sonido = audio · Subportadora = subcarrier · Reloj = clock · Entrada = input · Salida = output Pantalla = screen/display · Congelada = frozen · Lento = slow motion · Oscilador = oscillator

Clock Signal Captures — Communication & Signal, p. 5

All captures from 8 April 1997. This page is the primary empirical corroboration of the canonical clock values used in the Timing Reference. The technician labelled each scope shot in handwriting; component/pin assignments are from those labels.

| Signal | Component | Frequency | Amplitude | Notes |
| --- | --- | --- | --- | --- |
| A/V Clock IC | U7 / Pin 6 | 14.32 MHz | 2.28 V | Crystal input to U7 (MX8330MC). Sinusoidal. Corroborates f_xtal = 315/22 MHz ≈ 14.318 MHz within crystal tolerance. |
| Sub Carrier | U7 / Pin 8 | 3.579 MHz | 3.08 V | FSC output — colour subcarrier. Square wave. This is the signal fed to ENC-NUS (U5) SCIN pin via R13/R12 divider. Corroborates fS = 315/88 MHz. |
| Video Clock | U7 / Pin 1 | 48.64 MHz | 2.54 V | FSO output — VI master clock delivered to RCP (U9). Sinusoidal. Corroborates f_vi = 1071/22 MHz ≈ 48.682 MHz within tolerance. |
| RDRAM Clock IC | U15 / Pin 6 | 14.76 MHz | 2.32 V | PAL crystal input to U15. Corroborates PAL f_xtal = 17.734475 MHz (PAL crystal, not NTSC). |
| Master Clock | U10 / Pin 1b | 62.51 MHz | 3.82 V | CPU/RCP master clock. Labelled 'CPU as clock between U10–U9'. |
| RAMBUS Clock | U15 / Pin 3 | 250.2 MHz | 760 mV | RDRAM interface clock. Low amplitude (terminated). Not a timing derivation input. |

✔ Significance for Timing Reference The Sub Carrier and Video Clock captures are direct hardware measurements of the two frequencies from which all N64 refresh rates are derived. Both are consistent with the canonical fraction-based values in §3.1 within the ±30 ppm crystal tolerance documented in §3.7.1. These scope shots constitute independent empirical verification from a 1997 Nintendo service unit.

Video Signal Chain — Communication & Signal, p. 3

Four captures showing the signal path from the RCP digital output through to the final composite video output. All taken at 1GS/s. Annotations are handwritten in Spanish.

| Capture | What it shows |
| --- | --- |
| Tsync U4 / Pin 10 12.16 MHz | DSYNC signal at the VDC-NUS (U4) input. The scope annotation reads 'Video Sincronia — getting this signal; communication is established between video digital and the video digital converter.' Confirms DSYNC is a continuous clock-rate signal (not a pulse), consistent with the 4-beat VDC bus protocol. The ~12 MHz frequency is fvi ÷ 4. |
| SRGB Bus U4 / Pins 2–8 (1 GS/s) | The VDC digital data bus (D0–D6) captured at U4's input pins. Annotation: 'Video digital information between RCP and video-digital analogic converter.' Shows the 7-bit parallel digital video data arriving at the DAC. Amplitude 3.44 V active @ 3.3 Vpp. |
| Sync./BFlag U5 / Pins 5 & 6 (20 MS/s) | Two-channel capture: Pin 5 = CSYNC output; Pin 6 = BFP (Burst Flag Pulse) output. Both generated by U4 (VDC-NUS) and fed to U5 (ENC-NUS). BFP gates the colour burst window. The time delta annotation reads Δ 5.1 μs — the burst gate window duration. Annotation: 'Video Encoder'. |
| RGB NTSC U4 / Pins 17, 19, 21 (5 MS/s) | Analog RGB output from U4 (VDC-NUS) showing composite waveform with visible sync and blanking. Annotation: 'VDAC. Video Output.' Cursor markers show Δ 63.6 μs / 139 μs — consistent with NTSC line period (63.56 μs) and two-line interval. This is the analog signal feeding ENC-NUS. |
| NTSC P7 / Pin 11 (20 MS/s) | Composite video output at a test point (P7), with the colour burst region circled in blue by the technician — annotation 'Burst!'. Shows the full NTSC composite waveform: sync tip, back porch with burst, active video. This is the final output signal. |
| NTSC SC U5 / Pin 8 3.579 MHz | Colour subcarrier at SCIN (pin 8) of U5 (ENC-NUS). Amplitude 468 mV — attenuated from U7 pin 8 (3.08 V) by the R13/R12 resistor divider and C21 coupling capacitor documented in §4.2.1 and Figure 2e. This is the most significant video chain capture for our document. |

Diagnostic Flow Charts — N64 Repair Manual

The service manual's diagnostic cases map directly to system subsystems. Understanding the structure makes the flow charts readable.

| Case | Symptom / Subsystem |
| --- | --- |
| Precheck | Initial power-on check. Follows: Power LED → Video → Audio → Controller → Burn-In. Failure at Video+Audio with no other output → Case 1. |
| Case 1 | No audio or video output. Start here. Checks NMI signal (U6 pin 7) as the key diagnostic branch point. NMI active = PIF (U6) sensed a fault. Distinguishes: defective CPU (U10) vs. defective RCP (U9) vs. defective PIF (U6) vs. defective RDRAM (U11 / U14). |
| Case 2 | Video output fault (no colour, wavy picture, scrambled screen). Checks VSYNC clock at U9 pin 14, then SRGB bus at U4, then RGB/CSYNC/BFIN at U5, then FSC at U7 pin 8, then SCIN at U5 pin 8. Fault tree terminates at: replace U9 RCP / U4 VDAC / U5 RGB encoder. |
| Case 3 | Audio output fault (abnormal or absent audio). Checks AUDCLK/AUDLRCLK/AUDDATA at U9, then LOUT/ROUT at U1 (ADAC), then component integrity (C24–C26, C28–C31, R17, R21). Terminates at: replace U1 ADAC / replace U2 Audio Amp. |

⚠ Case 2 SCIN check — relevance to Timing Reference The Case 2 flow chart explicitly checks U5 RGB IC pin 8 = 3.58 MHz @ 0.5 Vpp as a discrete diagnostic step after verifying FSC at U7 pin 8. The reduced amplitude (0.5 Vpp vs 3 Vpp at U7) is the R13/R12 divider attenuating the subcarrier before injection into the encoder. The service manual independently confirms both the signal level and the hardware path documented in §4.2.1 and Figure 2e.

Start-Up Sequence — N64 Repair Manual, p. 5

The 13-step boot sequence is the complete picture of how the N64 initialises before program execution. Relevant to clock stabilisation and the MX8330MC 5 ms transient documented in §3.7.2.

| Step | Event |
| --- | --- |
| 1 | CPU initialises and outputs reset vector to RCP. |
| 2 | RCP addresses PIF for first byte of boot code via PCHCMD. |
| 3 | PIF returns first byte via PCHRSP. |
| 4 | RCP loads first byte, notifies CPU. |
| 5 | CPU sends RCP address for second byte. |
| 6–8 | RCP/PIF exchange second byte; RCP loads and notifies CPU. |
| 9 | Processors run boot program; PIF checks Joy Channel. (Note: controller must be active.) |
| 10 | RCP addresses program. |
| 11 | DRAMs initialise. |
| 12 | RCP transfers program data to DRAM. |
| 13 | Program starts. |

ℹ Clock stabilisation context The MX8330MC requires ~5 ms to stabilise after power-on (§3.7.2). This transient occurs during steps 1–4, before the PIF/RCP boot exchange completes. Active video output does not begin until step 13 at the earliest. The scope captures in Communication & Signal were all taken from a running system, well past this window.

Common Fault Patterns — Procedure for Testing N64, p. 1 (right column)

13 fault patterns with root causes, transcribed from handwritten technician notes. Some entries have marginalia indicating additional checks.

| # | Symptom → Root Cause |
| --- | --- |
| 1 | Frozen picture after 3–5 min → RCP (U9) defective. ('Processor of video and sound!') |
| 2 | No picture but sound → U4 defective. Video digital analog converter. |
| 3 | Slow motion picture and sound → crystal oscillator (X1) defective. |
| 4 | Too bright picture with no definition → check U4 (VDC) pin 17, amplitude < 700 mV. |
| 5 | Black screen with no sound can also be caused by CPU if everything else OK (NMI 3.3 V). |
| 6 | Power supply problem: once fixed, must wait 33 seconds before reset. |
| 7 | Clock signal can cause no picture and no sound. (Crystal X15 — check U15.) |
| 8 | Picture but no colour → Video Encoder (U5) defective; check subcarrier (short circuit). |
| 9 | Picture freezes after a few minutes → RCP may be defective. |
| 10 | Gets frozen after few minutes → RCP may be defective. |
| 11 | Before replacing RCP: check connector. (Oscilloscope note added.) |
| 12 | No picture and no sound might also be the clock (X2 crystal). |
| 13 | Frozen in certain games after few minutes → U14 defective. (RDRAM) |

ℹ Entries 3 and 7 are directly relevant Entry 3 (slow motion → crystal defective) confirms the clock-to-refresh-rate dependency: a degraded X1 crystal produces a lower f_xtal, which lowers f_vi, which slows the VI counter, producing a visibly reduced frame rate. Entry 7 (clock → no picture/sound, check U15) confirms the PAL crystal path is diagnostically independent — U15 failure affects PAL output, not NTSC.

Annotated Schematic — Procedure for Testing N64, p. 2

A full N64 system schematic (appears to be a pre-release or early production version) with extensive technician markup in three colours. This is among the most information-dense pages in the package.

| Annotation colour | Meaning / content |
| --- | --- |
| Pink circles | Voltage test points. Values annotated: Vref 1.9 V, Vref 2.05 V, 3.3 V rail, 3.5 V, 3.8 V. Used to quickly verify power delivery to each IC. |
| Blue/purple lines | Signal flow traces highlighting key bus paths: Rambus Channel, CPU AD32 Bus, AD16 Bus to cartridge slot, and the RCP-to-video signal path. |
| Blue text annotations | Component function labels: 'U10 = CPU · U9 = RCP · U8 = Buffer (interface) · U15 = Clock · U14 = RDRAM · U11 = RDRAM · U5 = Encoder Video RGB — converts video out 3V'. Also: 'CLK out 48.6 MHz 3.3 Vpp' at U9; 'Subcarrier 3' at U7. |
| Yellow circles | Audio signal path markers. Two yellow circles highlight the audio DAC region (U1) and the audio amplifier output (U2). |
| Handwritten notes (left margin) | 'Better to use DC coupling with digital signal' — oscilloscope coupling advice. Sketch of square wave vs. sinusoidal waveform illustrating the point. Note: 'AC with sinusoidal because of the negative 4T sides.' |
| Bottom margin | Clock values confirmed: 'CLK 511 14.7 3PP / in 14.31818 MHz' at U15 (PAL crystal input); 'CLK out 48.6 MHz 3.3 Vpp' at U9 pin 1b (master clock to CPU). Confirms both NTSC and PAL clock chains labelled. |

✔ U5 annotation of particular note The technician has labelled U5 as 'ENCODES THE RGB VIDEO INTO COMPOSITE VIDEO — converts video out 3V' with marginal notes 'Yout = higher than the same effect.' This is a live observation of the YOUT/VOUT output amplitude behaviour from the ENC-NUS encoder — the final stage in the signal path documented in Figure 2e of the Timing Reference.

NMI Signal — What It Is and Why It Matters

NMI appears repeatedly across all three documents as the primary diagnostic branch point. The Repair Manual makes it the centrepiece of Case 1 diagnosis. Understanding it prevents misreading the flow charts.

| Signal | NMI — Non-Maskable Interrupt |
| --- | --- |
| Source | PIF chip (U6). Pin 7. Asserted when PIF detects a fault condition during or after the boot sequence. |
| Normal state | Constant 3.3 V (logic high). Monitor for at least 10 seconds before declaring NMI inactive. |
| Active state | Oscillates — either irregular bursts at ~25 ms intervals (defective RCP, U9) or rapid 25 ms oscillation (defective PIF, U6). Scope capture in Communication & Signal p. 2 shows NMI at Ch1 Freq 162.8 kHz during a fault condition. |
| Why it matters | A defective CPU (U10) will NOT cause NMI. A defective RCP (U9) or RDRAM will. This is the critical distinction in Case 1: if NMI is active, the CPU is almost certainly not the problem. |
| Scope setup | Single acquisition, 25 ms/div, negative edge triggering, pre-trigger 1%, DC coupling. Probe PCHCMD (U6 pin 11) and power on. Compare acquired waveform to reference shapes A, B, or C in the Service Manual. |

Quick Reference — Pin-Level Diagnostic Checkpoints

Derived from Case 1, Case 2, and Case 3 flow charts in the Repair Manual. These are Nintendo's own specified verification points for a known-good NUS-CPU-03 board.

| Signal | Component | Frequency | Amplitude | Notes |
| --- | --- | --- | --- | --- |
| VSYNC | U9 / Pin 14 | 12.17 MHz | 3.3 Vpp | RCP video sync output. First check in Case 2 video fault diagnosis. |
| SRGB Bus | U4 / Pins 2–8 | active | 3.3 Vpp | Digital video bus from RCP to VDC-NUS. Verify active during video fault. |
| RGB | U5 / Pins 1–3 | 0–12 MHz | 700 mVpp | Analog RGB from VDC-NUS to ENC-NUS. |
| CSYNC | U5 / Pin 5 | 15.734 KHz | 3.3 Vpp | Composite sync output from ENC-NUS. |
| BFIN | U5 / Pin 6 | 15.734 KHz | 3.3 Vpp | Burst Flag Input — gates colour burst window. |
| FSC | U7 / Pin 8 | 3.58 MHz | 3 Vpp | Subcarrier at clock generator output. If absent → fix FSC clock. |
| SCIN | U5 / Pin 8 | 3.58 MHz | 0.5 Vpp | Subcarrier at encoder input. Attenuated by R13/R12 divider. If absent → check R12, R13, C21. |
| AUDCLK | U9 / Pin 6 | 1–8 MHz | 3.3 Vpp | Audio clock from RCP. Case 3 first check. |
| AUDLRCLK | U9 / Pin 10 | 22.05/32/44.1 KHz | 3.3 Vpp | Audio sample rate clock. |
| AUDDATA | U9 / Pin 7 | active | 3.3 Vpp | Digital audio data from RCP to ADAC (U1). |
| NMI | U6 / Pin 7 | steady | 3.3 V DC | Must be stable 3.3 V. Any oscillation = fault. See NMI section above. |
| PCHCMD | U6 / Pin 11 | varies | 3.28 V | PIF command channel. Used for NMI waveform comparison during boot. |
| Vref | U3 / Pin 4 | — | 1.91 V DC | Voltage reference. Cold reset captures show transient behaviour. |

N64 Timing Reference project  ·  Source cheat sheet compiled from Hard4Games leak (2020), Nintendo of Latin America service training package, c. 1997